Catscale 5x20 overhead sign fabrication

- Gather all materials

  - Spreader bars

  - Handhole covers

  - Lift pick brackets/lift bracket gussets

  - Chase nipples

  - Scabs

  - LAF irons

  - Corner gussets

- Attach scabs on top side of frame and above switch with rivets, drill screw holes with 9/64th drill bit opposing rivets

- Mark and cut/drill pole, camera, and drain holes on bottom of frame. Add rivet nuts for camera mount.

- Remove and label all pieces of retainer and c-cover. Keep switch and top center pieces of c-cover for later. Label frame.

- Flip frame. Find center (117”).  Use template to lay out pole centers/LAF irons/ light brackets. Drill drain holes.

- Use plasma torch and template to cut out handholes.

- Layout LAF irons and drill 9/16 mounting holes. Drill hole for chase nipples on top section in center of poles with 2” hole saw.

- Layout and bolt lift picks to inside of frame opposite top center LAF irons

- Tighten LAF irons. Make sure spacing is 16’ on center.

- Turn over and grind handhole covers smooth. Install handhole covers.

- Smooth off edges of spreader bars on belt sander and weld in place. Weld in lift picks at same time. Mark center of lift pick holes and transfer to top center pieces of c-cover.

- Turn over and install light bar brackets

- Stand up frame on saw- horses and clamp together in corners. Line up corners and tack weld on inside corner. Put in necessary brackets to hold interior steel.

- Put in spreader brackets between pole and interior steel. Insert poles and interior steel. Cross square frame (243 7/8).

- Weld steel (3 sides only)

- Weld aluminum.  All visible surfaces in corners, inside and outside. Weld in corner gussets.

- Flip sign over and weld any existing aluminum. Do not need to weld 2nd side of steel unless Highwind or Hurricane.

- Assemble switch. Make sure everything is over to paint.

- Double check all welds are completed. Pick up by bottom and put upside down on cart.